EET-225 Homework #5

Sr. Professor Wheeler

Instructions: This homework must be turned in within a flat 3-tab paper folder (no three-ring binders will be accepted). Answers must be written very neatly or typed. Use complete sentences when answering all questions. Where a problem involves a circuit, you must redraw the circuit as part of the solution, showing all indicated voltages and currents on the circuit diagram. Box or underline all final answers and show all work (see syllabus for example of homework standards).

Use 100 for both Beta DC and Beta AC for all problems in this assignment.

1. Explain how a common-emitter amplifier works by using the “emitter follows base” concept.

2. Give the formula that defines the voltage gain of a circuit. What units are used to denote voltage gain?

3. List the three BJT amplifier configurations. For each one, state where the input signal is applied, and where the output is obtained. List at least one characteristic of each configuration. (A table format is strongly suggested).

4. What is the voltage gain, input impedance, and output impedance of the amplifier below? Show all calculations.

5. Repeat problem 4, but change the load resistor RL to 4.7K. What happens to the voltage gain, and why?

6. What is the voltage gain of the amplifier below? Show all calculations.

7. If the emitter current in the amplifier of question 6 were to increase to 2 mA, how would the gain of the unit be affected?

8. Define the term swamping as applied to a common-emitter amplifier. Why is swamping usually desirable?

9. Calculate the lower cutoff frequency flco for the output coupling network of the amplifier in problem 4. What is the lowest frequency that is firmly coupled by the network in question?

10. In a coupling network having a 10K Thevenin resistance, what minimum capacitance value will provide firm coupling at a frequency of 100 Hz?

11. Construct the DC and AC load lines for the amplifier of problem 4. Calculate the compliance of the amplifier and use this value to determine the maximum power deliverable to the load.

12. Design a common-emitter amplifier to meet the following specifications. Show all calculations and the completed schematic of the unit.

10 <= AV <= 20 V/V
RL = 4.7 K
flco <= 100 Hz
Vpp >= 3 Vpp

13. Design a common-emitter amplifier to meet the following specifications. Show all calculations and the completed schematic of the unit.

20 <= AV <= 25 V/V
RL = 2.2 K
flco <= 10 Hz
Vpp >= 10 Vpp

14. Analyze the common-collector amplifier below and determine the following: Voltage gain, input impedance, output impedance, compliance. Draw the DC and AC load lines for the unit.

15. Design a common-collector amplifier to meet the following specifications. Show all calculations and the completed schematic of the unit.

AV  1 V/V
RL = 75 
Vcc = 5 V
Vpp (compliance) >= 2 Vpp
flco <= 30 Hz

16. Design a common-base amplifier to meet the following specifications. Show all calculations and the completed schematic of the unit.

10 <= AV <= 15 V/V
RL = 1000 
Vcc = 5 V
flco <= 1 MHz